Frame Relay

Model

| Presented by: | Dr.Sc. I. Ushakov, Chief Scientist Lead of the Project PhD I. Sokolov Team Leader S. Antonov Sr. Programmer |
| --- | --- |

June,  1995

MCI Telecommunications,  Richardson, Texas

GOAL OF  THE  PROJECT

THE GOAL OF THE PROJECT IS TO DELIVER A COMPUTER TOOL FOR A DESIGNER OF A FRAME RELAY NETWORKS

THE DESIGN OF THE FRAME RELAY NETWORK INCLUDES

- NETWORK CONFIGURATION

- LINK'S CAPACITIES

- NODE'S EQUIPMENT

- PVCs CONSTRUCTION

Statement of Work

DEVELOPMENT  AND  APPLICATION  OF

A  GENERALIZED PACKET-SWITCHED NETWORK ANALYSIS TOOL

List of Participants

| Igor A. Ushakov, Dr. Sc. | Chief Scientist., SOTAS, Inc. Lead & Scientific Adviser of the Project |
| --- | --- |
| Igor A. Sokolov, PhD | Chair of Department Russian Academy of Sciences (RAN) Leader of the team |
| Michael G. Konovalov, PhD | Senior Researcher, RAN |
| Sergei Ya. Shorgin, PhD | Senior Researcher, RAN |
| Sergei V . Antonov | Senior Programmer, RAN |
| Alexander P. Suprun | Senior Programmer, RAN |
| Elena I. Andreeva | Programmer, RAN |

AGENDA

- GENERAL INFORMATION

- INTERFACE

- CONSTRUCTION OF PVCs

- PERFORMANCE ANALYSIS

- SURVIVABILITY ANALYSIS

- EXAMPLES

- REPORTS

- CONCLUSION

Discussion

AGENDA

- GENERAL INFORMATION

- INTERFACE

- CONSTRUCTION OF PVCs

- PERFORMANCE  ANALYSIS

- SURVIVABILITY ANALYSIS

- EXAMPLES

- REPORTS

- CONCLUSION

GENERAL INFORMATION

OBJECT OF ANALYSIS

FRAME RELAY NETWORK

- HIGH SPEED TRANSMISSION BACKBONE

- FRAME RELAY SWITCHES  (FRSs)

- FRAME RELAY ACCESS  DEVICES (FRADs)

- USERS

- LINKS

THE PRINCIPLE OF TRANSMISSION

- HIGH SPEED TRANSMISSION OF FRAMES WITHIN FRN BACKBONE VIA PERMANENT VIRTUAL CHANNELS (PVCs)

BASIS:

- CCITT AND ANSI STANDARDS FOR FRAME RELAY

TASKS OF MODELING

- CONSTRUCTION OF PVCs BY CRITERION OF THE MINIMUM DELIVERY TIME

- PERFORMANCE ANALYSIS:  CALCULATION OF THE FRAME DELIVERY TIME AND PROBABILITY OF FRAME DELIVERY

- SURVIVABILITY ANALYSIS

- WHAT IF ANALYSIS

- PREPARATION OF REPORTS

- ARCHIVE OF PREVIOUS COMPUTER EXPERIMENTS

- HELP

INPUT DATA

- STRUCTURE

- TRAFFIC

- CHARACTERISTICS OF FRAMES

- CAPACITIES OF NODES (SPEED OF PROCESSING AND VOLUME OF INPUT AND OUTPUT BUFFERS)

- CAPACITIES OF LINKS

CONTENTS OF FRN MODEL

- ANALYTICAL MODEL

- DATABASE OF NODE'S AND TRUNK'S TYPES

- FRIENDLY USER'S INTERFACE

- GENERATOR OF REPORTS

- ARCHIVE OF MODELS WITH INPUT DATA

- PRINTING REPORTS AND SCREEN

- HELP

ANALYTICAL MODEL OF FRN

PREDESTINATION  OF  THE  ANALYTICAL  MODEL

- CALCULATION OF PVCs

- CALCULATION OF DELIVERY TIME

- CALCULATION OF PROBABILITY OF FRAME DELIVERY

- CALCULATION OF LINK'S AND NODE'S LOAD

ANALYTICAL MODEL IS BASED ON

- QUEUING THEORY

- OPTIMIZATION METHODS OF SEARCH OF EXTREMUM

DATABASE

DATABASE INCLUDES

- LIST OF NODE'S EQUIPMENT

	FR SWITCHES

	FR ACCESS DEVICES

- LIST OF TRUNK'S TYPES

DATABASE PERMITS USERS TO ENTER AND CORRECT DATA

USER'S INTERFACE

USER'S INTERFACE IS DESIGNED FOR FRIENDLY DIALOGUE OF THE USER WITH THE MODEL

INTERFACE ALLOWS USERS TO:

- DISPLAY THE STRUCTURE OF THE FRN AND

	MOVE

	ZOOM

	SELECT

	EDIT

OBJECTS ON THE SCREEN

- INPUT DATA

	NODE'S AND LINK'S PARAMETERS

	TRAFFIC

- RUN TASKS

- DISPLAY REPORTS

- TYPE TITLES, LEGENDS AND COMMENTS   (ON THE SCREEN AND FOR PRINTING)

- PRINT SCREEN AND REPORTS

AGENDA

- GENERAL INFORMATION

- INTERFACE

- CONSTRUCTION OF PVCs

- PERFORMANCE ANALYSIS

- SURVIVABILITY ANALYSIS

- EXAMPLES

- REPORTS

- CONCLUSION

CONSTRUCTION

OF  PVCs

Construction of PVCs

Increasing of Link’s Capacity or Number of Permitted PVCs

| Initial variant | For given network parameters No Solution |  |
| --- | --- | --- |
| Recommendations to increase number of permitted PVCs |  | are to increase capacity of links: Los Angeles-Houston up to 129,000 Kbps Washington-Atlanta up to 117,000 Kbps |

Construction of PVCs

Effect of Maximum Number Permitted of PVCs

| One permitted PVC | Two permitted PVCs |
| --- | --- |
| Three permitted PVCs | Networks with different values of the maximum number of permitted PVCs Origin = Seattle Destination = Miami |

AGENDA

- GENERAL INFORMATION

- DEMO OF INTERFACE

- DEMO OF CONSTRUCTION OF PVCs

- PERFORMANCE ANALYSIS

- SURVIVABILITY ANALYSIS

- EXAMPLES

- REPORTS

- CONCLUSION

PERFORMANCE  ANALYSIS

Performance Analysis

Impact of Traffic Intensity on the Delivery Time

| Network with Homogeneous Units: Link’s Capacity 140 Mbps Node’s Capacity 450 Mbps Load Origin-Destination 5 Mbps |  |
| --- | --- |

Performance Analysis

Distribution Function and Quantiles

Seattle-Miami

| Network with Homogeneous Units: Link’s Capacity 140 Mbps Node’s Capacity 450 Mbps Load Origin-Destination 5 Mbps | Value of Probability Quantile 0.00 0.00000 0.05 0.00200 0.09 0.00269 0.14 0.00301 0.19 0.00326 0.24 0.00347 0.28 0.00366 0.33 0.00384 0.38 0.00401 0.43 0.00417 0.47 0.00433 0.52 0.00448 0.57 0.00464 0.62 0.00480 0.66 0.00497 0.71 0.00515 0.76 0.00534 0.81 0.00555 0.85 0.00581 0.90 0.00613 |
| --- | --- |

Performance Analysis

Main Parameterses

Delivery Time (sec)                                                                    Probability of Delivery

|  | Mean Time | STD |  |  | Probability of¦ Frame Delivery | Probability of Frame Loss Due to Congestions | Probability of¦ Frame Loss Due Trunk's Errors |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Maximum (Miami- Chicago) | 0.0057 | 0.0021 |  | Maximum (Seattle-Denver) | 0.9999 | 0.0000 | 0.0001 |
| Minimum (Houston-Miami) | 0.0007 | 0.0002 |  | Minimum (Miami-Chicago) | 0.8757 | 0.1239 | 0.0004 |
| Average Time for Entire Network | 0.0024 | 0.0010 |  | Probabilities for the Entire Network | 0.9830 | 0.0168 | 0.0002 |

Link’s Load                                                                         Node’s Load

|  | Capacity | Load | Coefficient of usage |  |  | Capacity | Load |  | Coefficient of Usage |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Maximum (Atlanta-Washington) | 138960 | 115000 | 0.828 |  | Maximum (Denver) | 450000 | 260000 | 0.578 |  |
| Minimum (Houston-Miami) | 138960 | 30500 | 0.220 |  | Minimum (Miami) | 450000 | 115500 | 0.257 |  |
| Average for Entire Network | 138960 | 70544 | 0.508 |  | Average for the Entire Network | 450000 | 184116 | 0.409 |  |

AGENDA

- GENERAL INFORMATION

- INTERFACE

- CONSTRUCTION OF PVCs

- PERFORMANCE ANALYSIS

- SURVIVABILITY ANALYSIS

- EXAMPLES

- REPORTS

- CONCLUSION

SURVIVABILITY  ANALYSIS

survivability Analysis

Influence of Failures on PVCs

Sample: Origin = San Francisco, Destination = New York

| Initial variant | One Failure |
| --- | --- |
| Two Failures | Three Failures |

survivability Analysis

Changing of the Mean Delivery Time Depending on Failures

Network Fragments

| I II III IV |  |
| --- | --- |

AGENDA

- GENERAL INFORMATION

- INTERFACE

- CONSTRUCTION OF PVCs

- PERFORMANCE ANALYSIS

- SURVIVABILITY ANALYSIS

- EXAMPLES

- REPORTS

- CONCLUSION

EXAMPLES

Sample of special Analysis

Subnetwork. Local Burst of Traffic

| Subnetwork |  |
| --- | --- |

Sample of special Analysis

Subnetwork. Effect of Trunk’s Capacity

| Subnetwork | Link ‘User11-FRAD1’ consists of 1 trunk T-1 Link ‘User12-FRAD1’ consists of 1 trunk DS-0 Mean Delivery Time User11 to User_SW - 0.0057 sec User12 to User_SW - 0.0452 sec |
| --- | --- |

Sample of special Analysis

Effect of Trunk’s Type

| Links consist of 1 Trunk T-1 (1.544 Mbps) Mean Delivery Time 0.0057 sec | Links consist of 28 Trunks DS-0 (28x56Kbps = 1.568 Mbps) Mean Delivery Time 0.1167 sec |
| --- | --- |

CONCLUSIONS  AND  RECOMMENDATIONS

FRAME RELAY NETWORK MODEL  developed by SOTAS’s Moscow International Group of Operations Research (I.G.O.R.) helps MCI:

- Evaluate and Perform “What If” Analyses of Existing Networks and Systems

- Design, Plan, and  Analyze Prospective Networks and Systems

The Model:

- Is Interactive and User Friendly

- Finds Permanent Virtual Channels

- Calculates Many Measures of Operational Performance

- Yields Results in Meaningful Formats

Recommended Improvements to the Current Model Include Adding Switched Virtual Channels.

This  Model is the  Foundation  and SOTAS is  THE  Contractor to Build Tools to Evaluate:

- Asynchronous Transfer Mode (ATM) Networks

- SONET

- Dynamically Control Routing (DCR)

- Etc.